P80ADescription of automatically obtaining IP addresses

How to obtain the printer self-check page:

Load the paper ,close the printer cover,press the feed button and then turn on the power button,don’t remove your hand(feed button)until the paper comes out.

DHCP is turned on by default, that is, when the user is connected to the network cable, the printer can automatically obtain the IP address and print it out.

After the machine automatically obtains the IP address, DHCP is disabled and the obtained IP address is saved in the machine.。When DHCP is turned off, replace the network cable or network environment for the printer, and the IP address of the printer will not change.To change the IP address of the printer, set the DHCP status of the machine to on, or specify an IP address for the printer through the computer.

How to make the DHCP of the printer in the automatic acquisition status:

Restore the printer to the factory state, DHCP will be in the state of automatically obtaining the IP address;

How to restore the print status to factory status:

Open the upper cover of the printer (the printer should not be connected with a network cable), press the paper button of the printer for a long time, and power on the printer. The printer buzzer will ring three times first, and then long after 8 seconds. At this time, please remove the button that you press , and the printer has returned to the factory state.

The IP address of the printer is 192.168.1.168 (the default IP address is 192.168.1.168 when the printer leaves the factory)

How to specify an IP address for a printer through a computer:

Print a self-check page of the printer and find the IP address of the current printer on the self-check page. The computer environment and the current IP address of the printer must be in the same network segment. If they are not in the same network segment, set the IP address of the computer to the network segment of the printer. The following figure shows the WIN7 operation:

Set the IP address of the computer to the same CIDR block as the IP address of the computer:

Описание автоматического получения IP-адресов

Как получить страницу самопроверки принтера:

Загрузите бумагу, закройте крышку принтера, нажмите кнопку подачи, а затем включите питание, не убирая руку (кнопку подачи), пока бумага не будет извлечена. Там же указан IP-адрес принтера.

DHCP включен по умолчанию, то есть, когда пользователь подключен к сетевому кабелю, принтер может автоматически получить IP-адрес и распечатать его.

После того, как устройство автоматически получит IP-адрес, DHCP отключается, а полученный IP-адрес сохраняется в устройстве. Если DHCP отключен, замените сетевой кабель или сетевое окружение принтера, и IP-адрес принтера не изменится.

Чтобы изменить IP-адрес принтера, установите для устройства статус DHCP на "Включено" или укажите IP-адрес принтера с помощью компьютера.

Как перевести DHCP принтера в состояние автоматического получения данных:

Восстановите заводские настройки принтера, DHCP будет в состоянии автоматического получения данных.

Open the IE of the computer and assign a new IP address to the printer:

Откройте браузер компьютера, введите IP-адрес принтера и назначьте ему новый IP-адрес.